Sinclair ZX Spectrum Emulator 'Z80' v3.04  -  10/8/96  -  by G.A. Lunter��������������������������������������������������������������������Z80 - Frequently Asked Questions1  - How to print the diagram of the tape-interface     as shown in DIAGRAM.Z80, or make other screendumps.2  - How to LLIST and LPRINT to a printer3  - How to build the tape-interface4  - What to do if the Zandbergen joystick doesn't work correctly5  - How to contact the author6  - Manic Miner and Jetset Willy run too fast!7  - How to save a .SCR snapshot to tape or .TAP file8  - Printing problems with Tasword 1289  - What to do about an IN 0 crash?10 - I have a .TAP file, but it doesn't work                            ��������������������������������������������������������������������ad 1. How to make a screen-dump on a printerWhen the Interface 1 appeared, it was possible to attach a serial printerto the RS232 port of the Interface 1.  The emulator can redirect the RS232to the serial port of the PC, a file or a parallel (printer) port.The SamRam can make a screen-dump on a dot-matrix printer. It isprogrammed for a Star SG-10.  The Spectrum 128's COPY command sends itsoutput to the "p" printer channel, also an RS232 channel.  This will workfor Epson-compatible printers.  The ZX Printer's COPY output can betranslated into graphics data understandable for Epson and HP-PCLcompatible printers, and finally, the DISCiPLE has also on-board screendump routines.Example 1: Screen-dump for Epson or HP-PCL compatible printers, via   the standard COPY command.   Select the printer port to use for output with -XL 2 for example, to   select LPT2 for output.  Select Epson or HP-PCL output format with -0e   or -0h.  Then simply type COPY (in 48K or SamRam mode, not in 128K   mode).Example 1: Screen-dump for Star SG-10 compatible matrix printer using   the SamRam.   Redirect the Spectrum Interface 1 RS232 output to LPT1 by pressing F4,        O, L, 1.   Load a .Z80 file, and wait for the picture you want to print to appear.   Switch to SamRam mode by pressing F10, H, 3, CTRL-ENTER, and generate an        NMI by pressing X, N.  If you're already in SamRam mode you can        simply press F5.   Press P, then 1 to print the screen.Example 2: Screendump for EPSON compatible matrix printer using the   COPY command of the Spectrum 128   Redirect the RS232 output to LPT1 by pressing F4, O, L, 1.   Load a .Z80 file, and wait for the picture you want to print to appear.   Save it as a .SCR screendump by pressing F10, X, S, name   Switch to Spectrum 128K mode (F9, 5, ENTER)   Choose '128K BASIC' from the menu, and type PAUSE 0:COPY, and press        ENTER once.   Press F10, X, L, name, to load the screen snapshot back into memory.   Go back to the emulator (ESC, ESC) and press any key to start printing.Example 3: Screendump for all other printers, using DOS utilities   Follow the steps from example 2 up to the saving of the .SCR        screen snapshot.   Leave the emulator.   Run CONVERT on the .SCR file to generate a .GIF or a .PCX file, for        example                CONVERT b g screen.scr screen.gif,   or                CONVERT b x screen.scr screen.pcx   Find another program that can print the .GIF or .PCX fileExample 4: Screendump for Epson or HP PCL compatible printers using   the screendump routines of the DISCiPLE   Redirect RS232 output to LPT1.   Switch to 48K + DISCiPLE mode (or 128K + DISCiPLE) and select the ROM        with the pre-loaded D.O.S. for Epson or HP PCL compatible printers   Type COPY SCREEN$ 1 for a small screen-dump (black/white but with regard        to colors)   Type COPY SCREEN$ 2 for an A4 dump with dithered grey-tones.��������������������������������������������������������������������ad 2. How to LLIST and LPRINT to a printerThere are several ways to print things.  The standard ZX Printer output canbe converted to graphics output in Epson or HP-PCL format; for this, selectoutput port with -xl and output mode with -0e or -0h, then simply useLPRINT, LLIST and COPY to output to the "p" stream (on 48K Spectrum orSamRam; not on the Spectrum 128K).All output to the Interface I's RS232 channel "b" and "t" (for raw and textoutput respectively; use "t" for listings) and the Spectrum 128's RS232channel "p" and the parallel printer interface of the DISCiPLE/+D can berouted to the printer port, a serial port or a disk file.  Thus, by OPEN#3,"b", "t" on the 48K Spectrum with Interface 1, or OPEN #3,"p" on theSpectrum 128, all data sent to channel 3 (i.e. printed by LPRINT or LLIST)can be routed directly to a printer.The normal ZX Printer "p" channel is automatically (by an interruptroutine...) converted into the DISCiPLE/+D parallel printer output channelby typing POKE @11,0.  POKE @11,1 resets the "p" channel to the defaultagain.As said above, you have to redirect the output of the Interface 1 RS232.In the emulator, press F4, O, L, 1.  Like on the real spectrum withInterface 1, if you want to print to the Interface 1 RS232, you have toopen the #3 channel.  In Spectrum Basic, type: OPEN #3,"b"  (or OPEN #3,"t"for basic listings).  Press SHIFT-ALT 4 for OPEN #.In this way you can print anything as long as your spectrum programsupports printing to the Interface 1 RS232 and your printer type, or to theZX printer.  For example, it is perfecty possible to print a textfile onyour printer with Tasword 3, or make a printout with Art Studio.�������������������������������������������������������������������ad 3. How to build the tape interface.The diagram of the tape interface is in the program DIAGRAM.Z80. It is easyto build, and inexpensive, since only passive components are used.If you do not want to build the interface yourself, you can order one forBP 14.50 with B G Services.  For the address, see the Registering sectionin the documentation file, or the REGISTER.DOC file.�������������������������������������������������������������������ad 4. What to do if the Zandbergen joystick doesn't work correctlyThe Zandbergen joystick interface did not work at all in version 2.01.  Itworks properly in V3 again.  Sorry to have kept you waiting so long...To a fully implemented pc-joystick port, two analogue joysticks can beattached (on one plug!).  A number of multi-io cards can handle only oneanalogue joystick.  However, the Zandbergen joystick needs a fullyimplemented joystick port. If only two directions (out of four) work, yourjoystick port is not usable.  Use an analogue pc-joystick instead.The joystick port of a Sounblaster is fully implemented and ok to use.If you have more than one joystick port in your pc, you have to disableone.�������������������������������������������������������������������ad 5. How to contact the author.You can contact me by normal post:Postal address:        Gerton Lunter        P.O. Box 2535        9704 CM  Groningen        The Netherlands.If it is REALLY urgent you can Email:z80help@bgserv.demon.co.ukAny queries which require my help will be forwarded to me.�������������������������������������������������������������������ad 6. Manic Miner and Jetset Willy run too fast!True. Most of the time these games are either producing sound or executingan LDIR instruction to update the screen. Since this instruction can beemulated with a single REP MOVSB instruction of the 8086, emulation issuddenly very fast. Without sound, Manic Miner and Jetset Willy were twoof very few games that were actually playable on a 8 MHz PC, and of courseI don't want to slow down the emulator on slow machines!If you turn the LDIR emulation switch on, the LDIR instruction will behavesomewhat differently, important for well-protected programs. As a sideeffect, it will then also run at the right speed.�������������������������������������������������������������������ad 7. How to save a .SCR snapshot to tape or .TAP fileFirst make the appropriate choises in the tape menu, F7.  Then type SAVE"name" SCREEN$.  Press ENTER, and then press F10 in the one second gapbetween header and data block.  When saving to tape you can easily see it;when saving to disk, after pressing ENTER give the emulator a split secondto save the header to disk before pressing F10.Now load the .SCR snapshot into memory, and continue saving.�������������������������������������������������������������������ad 8.  Printing problems with Tasword 128Tasword 128 does not print correctly via the "p" channel; at least, myversion of Tasword 128 doesn't.  The problems can be circumvented easily byopening stream 3 for output to channel "b", i.e. OPEN #3,"b", instead ofusing channel "p".  A better solution is to remove the bug.  Load Tasword128, turn on Microface 128 emulation, hit NMI (F5), choose T, then H forHex, address 9DD3, and poke the values 01 FD 7F ED 79 32 5C 5B FB C9 intothe respective addresses (pressing ENTER twice after each value); then Qand R.�������������������������������������������������������������������ad 9.  What to do about an IN 0 crash?The Interface I often crashes if the 'Microdrive Data' port is accessedwhen the microdrive is not running.  Entering PRINT IN 0 in Basic willusually hang the Spectrum.  So, if a program reads from the 'MicrodriveData' port when it shouldn't, the emulator lets the user know that anordinary Spectrum would have crashed at that point.Some programs did indeed crash if an Interface I was attached; JetSet Willyis an example (just wait until it pauses, then press a key).  To use theseprograms, simply select the hardware mode 48K (or 128K) Spectrum withoutInterface 1.  In this mode the emulator won't bother about programs readingfrom port 0.��������������������������������������������������������������������ad

10.  I have a .TAP file, but it doesn't workCould be different things.  The Warajevo emulator uses a tape file formatdifferent from Z80's, but uses files with the same .TAP extension.  Checkthis first.  Wavajevo comes with a utility to translate its .TAP format toZ80's.Some programs need the R register emulation to be switched on.  Someprograms will not load with an Interface 1 emulated.  Some programs areSpectrum 128K programs without saying so when loaded in an 48K machine.Switch to 128K mode (without Interface 1 to avoid troubles there) and tryagain.Some programs use the standard format on tape, but use their own loadroutine.  Because Z80 traps the ROM and does not emulate the bips andclicks of the data, this will not work.  Convert the .TAP file to a .VOCfile using TAP2VOC, and play this file back to the emulator.