ELECTRON

SECOND

PROCESSOR

# Speed up your Electrons and watch your memory expand with a 6502 second processor, designed by John Wike with Electron owners in mind.

This article describes the addition of a second processor board to an Acorn Electron, making 30K bytes of RAM available to BASIC (60K to machine code), and giving an

increase in processing speed of up to three times. The hardware will be described this month and the software next month, together with a complete assembly listing.

# What about the others?

Although the term 'second pro-cessor' is usually associated with Aorn and their 'Tube' system, multiprocessor designs are found in several microcomputers in the business and scientific markets. Even the Sinclair QL contains two microprocessors, one to handle input/output and the other to do the computing. So, although the circuit shown here is designed specifically for the Acorn machines, the concept is generally applicable.

It is relatively straightforward to design a circuit board with a pro-cessor and some RAM on it, and to interface it with an existing computer system. The real problem is the software, machine code of course, to handle the new hardware.

Benchmark       Mode 6		    Mode 0	       E2P	    (BBC)

1	              0.93		       2.11	       0.68	      0.8

2	              4.01		       9.35	       2.99	      3.1

3	            11.54		     26.97	       8.43	      8.3

4	            12.27		     28.86	       8.95	      8.7

5	            12.85		     30.15	       9.37	      9.1

6	            19.51		     45.72	     14.35 	    13.7

7	            30.09		     69.88	     22.24 	    21.3

Table 1 Benchmark timings for the Electron with and without E2P.

As the host machine probably has the screen RAM within its memory map, it must be assumed at it will retain the input/output handling functions. This means that the language (usually BASIC II) operates in the second processor.

It is necessary to know how to intercept the input/output routines (PRINT, INPUT, SAVE, LOAD, etc.), so that the data will be transferred to or from the second processor's memory instead of the host's. Routines can then be written to reside in each processor's memory and allow them to communicate with each other transparently, so that the user will not be aware of any difference in operation from the basic machine.

All this sounds involved, but  given a machine that is well supported by reference material and ROM listings, or your own skill at disassembly, it is by no means impossible. So if you are interested have a go!

# 2P or not 2P?

The owner of an Acorn machine does not need to worry about the foregoing because this article will cover all the ground. He or she will however have to decide whether it is worthwhile adding a second processor to the system. There are several advantages to balance against the effort involved:

# Speed

The benchmark system has gained widespread acceptance as a qualitative assessment of the processing speed of a computer. For a full discussion of benchmarks the reader is referred to the February 1985 edition of Computing Today. Each test consists of 1000 iterations of specific instructions, the times for which are given in Table 1. Also included for interest are the timings for the BBC computer, taken from the Computing Today article. In Mode 6 the unexpanded Electron is approx-imately 50% slower than the BBC, and in Mode 0 it is 250% slower! With the E2P board fitted it is approximately the same as the BBC in all modes.

# Memory

The display memory in the Electron can consume between 8K of RAM in Mode 6 and 20K of RAM in Modes 0, 1 and 2. Add to this the 3.5K used as operating system workspace, up to 1.5 K for user-defined characters and an extra 3.75K if the Plus 3 disc drive is fitted, and out of a total of 32K there might only be 3.25K available for programs. The E2P board contains 64K RAM, 30K of which can be used from BASIC whatever the configuration. Machine code programs can use a massive 60K.

# Processor

The first requirement of the design was that the hardware and software should react with the Electron operating system in the same way as the official 'Tube'. This is a ULA with eight bidirectional registers, addressed at FCE0h to FCE7h, of which seven are used by the support software and only one, at FCE5h, is accessed directly by the operating system for data transfer during, for example, LOAD and SAVE. So the circuit must detect accesses at FCE5h and interrupt the second processor to allow it to pass the required data. The other registers can be at any convenient address, since they have their own support software.

The author’s

The author’s prototype second processor board (some changes have been made in the final version).

The only storage device on the board is the RAM. The top 256 bytes of that are accessible to the Electron, so that several locations can be used as the bidirectional registers. Also, as this is the area where the 6502 goes at Reset, the Electron can control its reset and transfer sufficient code there beforehand to allow it to "boot up'. After that the rest of its operating

system can be sent via the data byte at FCE5h.

When deciding where in the Electron memory map to locate this 256 byte block, it was remembered that sideways ROMs are given the opportunity to initialise themselves at BREAK and to declare themselves during the *HELP command. The block is therefore addressed as a sideways ROM and the first eleven or so bytes are taken up with the necessary data for it to be recognised by the operating system. They also contain a jump instruction so that the 'ROM' software can be in the main program in the Electron RAM.

In order to refresh the dynamic RAM the processor is interrupted every 1ms and a specific routine scans 128 bytes in 64us. On alternate interrupts it scans another 128 bytes to include all the rows in the RAM. This results in a time overhead of 6% which is considered acceptable by the author. Because the refresh is software controlled there is no facility for a hard reset of the processor. Instead, the 'sideways ROM' routine issues an initialisation request on BREAK.

# Interfacing

The board is designed to slot into one of the cartridge sockets on the Plus One interface unit, which provides some of the address decoding. For those people without a Plus One, a circuit is shown allowing connection to the basic Electron.

Current consumption of the board is about half an amp, which the author's machine was able to cope with. If a lot of other devices are drawing power, it may overload the

supply. A link (LK1) is provided to disconnect the 5 volt line from the edge connector and an alternative supply can then be connected to the board.

# Construction

Construction of this project is straightforward but you are recommended to use a fine tipped soldering iron, and to check the board closely to see that no stray bits of swarf or solder are shorting tracks.

As this is a double sided PCB and is not plated through, the first thing is to insert all the links and solder them on both sides of the board. Take special care not to miss the ones underneath ICs as these will be impossible to fit afterwards.

Next fit all the ICs except the RAMs and the processor, soldering their leads on the bottom, top or both, as necessary.

Next fit the resistors, capacitors and diode. Some of these components need to be soldered on both sides of the board.

Now fit the sockets for the RAMs and processor. Use insulating tape to protect the through-board links before inserting the sockets.

If you intend to power the board from the Electron's 5 volt line fit the

link LK1. Otherwise, connect the external supply wires to points A (0 volt) and B (5 volts).

Finally, insert the RAMs and processor into their sockets.

If you do not have the Plus One unit you will now have to construct the interface circuit. This could be done on Veroboard and then connected, along with the second processor board, to the Electron with a short length of ribbon cable and a 25 pin double-edge connector.

## HOW IT WORKS

The second processor is reset at switch on by latch IC8a with C4 and R12. Diode D1 ensures that C4 will be discharged quickly at switch off.

IC8 is a quad S-R latch with Set overriding Reset. Sections c and d are used to provide extra AND functions.

Decoding of the host processor address bus is performed by IC17, IC8d and IC1f. When either the sideways ROM or FCE5h are accessed, the output of IC17a will go low. This signal enables the data bus buffer, IC18. It also disables the second processor address multiplexers IC2 and IC3, after being inverted by IC1a, and it operates the control line multiplexer IC5. IC5 determines which processor's R/W and clock signals will be applied to the RAM - when Pin 1, select, is low the second processor clock and R/W times are effectively disabled.

The second processor 2MHz clock is generated by IC4 divifing down the master 16MHz oscillator. Further division takes place in IC4 and IC6 to give the 1ms NMI signal. Note that as the processor only responds to negative edges there is no need to provide short pulses.

# Setting up

Before switching on, check the board very carefully for shorted tracks and the orientation of ICs, diodes and electrolytic and tantalum capacitors. To ensure that the Electron will will not be damged check every contact on the edge

connector with a meter for shorts to either the 0 volt or 5 volt supply

lines.

Connect the board and switch on. You should get the start-up message as usual. If not, then switch off and check again for shorts. If you can get hold of a double-beam oscilloscope, connect a probe to pin 9 of IC8 and enter and run the following program:

10 A%=?&FCE5:GOTO10

You should see on the 'scope a negative going pulse 15ms wide. If it is a different width adjust the values of  R11 and/or C5 to get it as close as possible.

The output of IC17a is combined with the host processor clock in IC8c, which triggers the 15us IRQ monostable IC8b. The monostable is inhibited during ROM selection by the input at pin 11 so that it will only operate during an access at FCE5h. The monostable output is also fed to IC8a to clear the second processor reset.

The output of IC8a goes via IC1b to set the second processor cock in phase with the host’s during host access. Because the clocks are in phase, the control multiplexer IC5 can be guaranteed to switch when they are both low.

Tri-state buffer IC7 with resistors R3-R10 performs the address multiplexing function for host processor addresses. It is enabled during the first part of a host access to give the row address for the RAMs, then it is disabled and the resistors provide the column address. If it is a ROM address, IC8d output will be high and page FFh will be accessed. For address FCE5h, IC8d will be low and page FEh will be accessed.

The RAS signal to the RAMs is provided by IC1c and the CAS signal by IC1d and IC1e from the delay circuit R2-C3.

Now connect one input of the scope to IC1 pin 1 and the other input to IC1 pin 6. The negative going edge of the signal on pin 6 should occur between 120 and 150ns after the positive going edge at pin 1. Adjust R2 and/or C3 as neccesary.

Before trying the system in earnest you will have to switch off then on again in order to hard reset the second processor.

The necessary software will be included in next month's article, but

in the meantime a copy of the assembly code is available on tape from the author. Alternatively, if you send a Plus Three disc it can be stored on that together with the

#### PARTS LIST

#### INTERFACE

RESISTOR

R13      470R

CAPACITOR

### C15      220p

SEMICONDUCTORS

IC20     74LS30

IC21     74LS27

IC22     74LS20

IC23     74LS139

IC24     74LS74

MISCELLANEOUS

Veroboard, 22-way double sided socket to fit E2P edge connector, 25-way double sided socket to fit Electron edge connector.

machine code as a !BOOT file so that pressing shift-break will automatically boot in the second processor. See Buylines for details.

BUYLINES

##### PARTS LIST

##### MAIN BOARD

RESISTORS (all 1/4 watt)

R1,2,3,4,5,6,7,8,9,10    470R

R11                               390R

R12                               4k7

CAPACITORS

(all ceramics unless stated)

C1                             470pF

C2                             33pF

C4                             100uF electrolytic

C5                             47nF

C6,7,8                       47u 10V tantalum

C9,10,11,12,13,14    100nF

SEMICONDUCTORS

IC1                            74LS14

IC2,3                         74LS257

IC4,6                         74LS393

IC5                            74LS157

IC7,18                       74LS245

IC8                            74LS279

IC9,10,11,12,13,       4164-15/4864P-2

14,15,16

IC17                          74LS21

IC19                          6502A (2MHz)

D1                             1N4148

MISCELLANEOUS

40 pin DIL socket, 8x16 pin OIL sockets, wire for links.

All the components are available readily from advertisers in ETI. The PCB and software are available from the author, John Wike, at 9, Lon-y-Garwa, Caerphilly, Mid-Glamorgan.  The  price of  the PCB  is

£12, software on tape is £3.50, and on your disc £2.00, inclusive of postage, if you send a disc please state whether you wish to have the !BOOT file put on it.

ELECTRON

SECOND

PROCESSOR

It doesn't take ESP to know that we're dealing with the software for John Wike's Electron add-on.

Having described the hardware for this project last month, it is now time to consider the software. When RUN the program creates a 2K machine code file on tape or disc called E2PCODE. This latter is what must be *RUN to operate the second processor. Alternatively, if you have a disc it can be renamed as !BOOT and run using shift-break.

The listing given in this article has been produced using a formatting program to line it up nicely. If you intend to enter it yourself you must leave out all the spaces (except those after the'.' |abels) or it will not fit into memory.

###### Address I/O (2P)	Function			Possible Values

800D(FF0D) h	Pass Instruction to 2P	8 h  Start memory read(0)

9 h  Start memory write(1)

C h  Run program(4)

28 h Stop memory read/write

40 h Handle Event code

80 h Pass BRK error message

C0 h Set/Reset Escape flag

FF h Initialise 2P (Reset)

8013(FF13) h	Pass OS call to I/O	LSB Address of I/O OS routine

8014(FF14) h	Flush Oswrch buffer flag	Equal to 801E(FF1E) h

8019(FF19) h	Status for I/O to2P	40 h Data is text

80 h Data is not text

C0 h End of data

801A(FF1A) h	Data for I/O to 2P

801B(FF1B) h	Status for 2P to I/O	As for 8019(FF19) h

801C(FF1C) h	Data for 2P to I/O

801D(FF1D) h	Oswrch buffer remove pointer  Pointers equal if buffer empty

801E(FF1E) h	Oswrch buffer insert pointer  	“

Table 1.    Communication protocols.

The beauty of the Acorn machines is that in order to intercept the input/output operations of any language it is only necessary to modify eleven well-documented operating system (OS) routines. Detailed descriptions of these can be found 'The Acorn Electron Advanced User Guide' by Holmes and Dickens, and 'The Advanced User Guide for the BBC Micro' by Bray, Dickens, Holmes. The operating system calls are the same for both machines and while one book also covers the special hardware in the Electron, the other is more readily in the shops.

Memory Usage

In this article the Electron processor will be referred to as the I/O processor because that is its function in the new environment. The second processor will be called 2P for short.

The E2PCODE program loads intp addresses 2800h to 2FFFh in the I/O memory, as shown in the map. This is below the highest resolution screen. It uses memory (again I/O) at 0 to 70h and 400h to 40Bh so it is important that no user programs corrupt these three areas of I/O memory.

The main program is in three sections. Lines 190 to 4880 run where they are in I/O memory. Lines 5930 to 7250 are copied to the 256 byte sideways ROM area and exist at 8000h in I/O memory and FF00h in 2P memory. Finally, lines 7300 to 11100 are passed to 2P memory at F800h to FAFFh.

# Communication

The processors communicate with each other by way of various locations in 2P RAM which are used as status and data registers. Their functions are listed in Table 1 and, together with the 28byte Oswrch buffer, they overwrite the 2P reset routine in lines 6020 to 6230. As

there is no hard reset that routine is not needed again anyway. With all the registers, except the buffer pointers, a zero value indicates that the message data has been received and acted on.

# Facilities

This implementation has the ability to reset into the I/O processor by pressing B-break, ie hold down the B key while pressing BREAK. Programs can then be developed in the I/O memory without switching off. In I/O HIMEM is set to 2800h to protect the program. In 2P HIMEM is 8000h and PAGE is 800h.

If you execute *HELP while in 2P you will get the message "E2P 0.1". This does not appear if you are in the I/O.

Just one word of warning. Do not use any EVENT handlers in 2P that call OS routines. If you do the system will most probably hang up.

You will have to find out by trial and error which programs will work with a second processor. Anything that accesses screen memory or hardware directly will not work, since this can only be done by OS commands across the interface.

HOW IT WORKS

I/O OPERATION

When the program is run the I/O pro-cessor enters the cold start routine at line 1250. At lines 1350 to 1630 it checks each ROM number from 7 down to 0 for a block of RAM of 256 bytes or less. If it finds one it assumes that it is the E2P card and stores the ROM number in location 'this ROM', if not it returns to the language ROM and prints the error query, ??.

If the ROM is not logged in already (1640-1670), it then cycles through the RAM (1690-1720), loads the RAM (1740-1770) and clears the 2P reset (1780). The RAM is then cycled to refresh it (1800-1820) until the 2P signals that it is ready (1830-1840). It then initiates a 2P memory write and transfers the 2P operating system routines over (1860-2040) before soft resetting itself (2050).

During I/O reset the operating system calls the sideways ROM service routines at 8003h (line 5940) with the Tube reset code FEh in the accumulator. This is detected at line 2350 and the Osbyte vector is changed (2480-2570) to allow the new routine (3000-3140) to set HIMEM and inhibit the memory clear facility. Lines 2590-2670 check for the B key (64h in line 2630). If it is not pressed the Tube presence at Osbyte EAh and the IO/second processor flag at isec are set. The 2P is initialised (2700-2740). The vectors are changed, the soft character set is exploded, the filing system status is set and the sign on message is printed (2670-2920) before leaving the service routine.

Before the I/O can  communicate with the 2P it must select the correct sideways ROM location. This is done with the routine at 3230-3320.

If the Tube presence flag is set during I/O reset the operating system will not start up a language but will jump instead to location 400h. This has been loaded at line 2690 (via lines 380-430) with the code in lines 210-240. So it will enter the warm start routines at line 4120. Pointers are set up to transfer the current language to 2P memory either at 8000h (4130-4180) or at a specified relocation address (4190-4300). If the language has been selected by a * command (4320), or if this is a cold start (4330-4340), or if it is a hard reset (4350-4370), and it is not Plus One control ROM at 0Ch (4380-4410), then the language will be transferred to 2P memory (4430-4550). The language will then be started up in 2P by issuing a Run Program command (4590-4680).

The actual routines to give instructions to 2P start at line 3340. The operating system will enter these via the relocated line 230 at 406h, but they are called directly within this program. Lines 3340-3500 allow different filing systems to claim the 2P interface by entering with their Tube ID number plus C0h in the accumulator and waiting for the carry flag to be set. They release the interface using 80h plus the ID number. The read, write and run instructions (0, 1 and 4 respectively) enter with YX pointing to four consecutive locations which hold the action address for the 2P. This address is sent in lines 3550-3660 then the actual instruction number Ored with 8 so that it cannot be zero is sent (3690-3710). If the instruction is to run a program, the I/O processor will enter its main loop, otherwise it will return (3740-3820).

The main loop routine (5540-5880) maintains the rotating Oswrch buffer remove pointer at 801D(FF1D)h. If the buffer is not empty its contents are printed. If it is empty the status register at 8013(FF13)h is examined and an OS call is executed if desired.

2P OPERATION

When the 2P reset is cleared it jumps via its reset vector at FFFCh (7240) to line 6020. Here it loads a 128 byte RAM refresh routine twice into FB00h to FBFFh. This consists of 'compare accumulator immediate' instructions (C9h), which are two bytes long and take two cycles to execute. Thus it will access 128 bytes in 128 cycles, or 64 microseconds. The last few bytes of the routine contain lines 6170-6230 which modify the calling routine to scan the other 128 bytes next time round (6170-6190) and check the status register at 800D(FFOD) h to see if any instructions need to be executed.

Normally the NMI routine (6400-6420) will call the cycle routine (6350-6380) to perform the refresh. However while the I/O is writing or reading 2P memory via the data register at FCE5(FEE5)h the 2P must respond quickly to the IRQ line. So the NMI is disabled and the 2P goes into the loop (6680-6700) where it is continually cycling to do the refresh. The read/write function is performed by modifying the three locations at ivec (6270) in the irql routine (6250-6330) to read or write the specified start address (6470-6550). If it is a read the first location is read by a software interrupt (6610-6620).

The other instructions are looked for in the test routines (7460-8310) and the appropriate action taken. The 2P OS routines (8330-9720) interact with their I/O counterparts (450-1230) via the register at 8013(FF13)h. Some routines need to pass over the processor A,X and Y registers. Some need to pass text and some need block data. These are handled by the routines at lines 4700-5360, 6730-6780, and 9740-10320. The Osword routines transfer different amounts of different size blocks. The tables at 10480-10910 are

used to determine how much to send.

The 2P Osbyte routine checks for the memory functions 82h to 84h (8420-8540). these give the machine high order address (FFFFh for I/O, 0000h for 2P), PAGE, HIMEM, and HIMEM in other Modes, in that order. It also checks (8560-8640) whether the Oswrch buffer is being flushed (Osbyte DAh,0,0) and sets the flag at 8014(FF14)h.

Listing of Electron program to run second processor. 			       Copyright John Wike, 1985.

50 REM AS SPACE IS SHORT WHEN

60 REM USING DISCS, THE CODE IS

70 REM ASSEMBLED INTO SCREEN

80 REM MEMORY BEFORE SAVING.

90 :

100 MODE6:VDU28,0,24,39,12

110 :

120 rgX=&F0:txt=&F2:rpl=&F4

130 blk=&96:num=&98:tmp=&99

140 cmd=&9A

150 :

160 U%=&2800:V%=&6500:HIMEM=V%

170 A%=U%:B%=U%

180 FOR I=4TO6STEP2

190 O%=V%:P%=U%

200 [OPT I

210 JMP cold

220 :

230 .cod400

240 JMP warm

250 JMP esc

260 JMP instr

270 .osjmp

280 JMP osrdch

290 :

300 .pload

310 LDA (0),Y

320 INC 0

330 BNE pload2

340 INC 1

350 :

360 .pload2

370 STA &FCE5

380 :

390 LDY #7

400 .delay

410 DEY

420 BNE delay

430 RTS

440 :

450 .codld

460 LDX #&C

470 .codld5

480 LDA cod400-1,X

490 STA &3FF,X

500 DEX

510 BNE codld5

520 RTS

530 :

540 .osrdch

550 JSR &FFE0

560 .osrd1

570 JSR regso2

580 .osret

590 LDA #0

600 STA &8013

610 .anrts

620 RTS

630 :

640 .oscli

650 JSR datin

660 JSR &FFF7

670 JMP osret

680 :

690 .osbyte

700 JSR regsin

710 JSR &FFF4

720 JSR regsout

730 JMP osret

740 :

750 .osword

760 LDA &8016

770 BNE oswd5

780 JSR datin

790 STX 0

800 STY 1

810 TXA

820 TAY

830 JSR &FFF1

840 JSR regso3

850 JSR osret

860 BCS anrts

870 JMP txtout

880 :

890 .oswd5

900 LDY #&20

910 JSR datin1

920 LDX #&20

930 LDY #0

940 LDA &8016

950 JSR &FFF1

960 JSR osret

970 JSR regsin

980 JMP blkout2

990 :

1000 .osargs

1010 JSR datin

1020 JSR regsi1

1030 JSR &FFDA

1040 JSR osrd1

1050 JMP blkout

1060 :

1070 .osbget

1080 JSR regsi1

1090 JSR &FFD7

1100 JMP osrd1

1110 :

1120 .osbput

1130 JSR regsi1

1140 JSR &FFD4

1150 JMP osret

1160 :

1170 .osfind

1180 JSR regsi1

1190 BEQ osf5

1200 PHA

1210 JSR datin

1220 PLA

1230 .osf5

1240 JSR &FFCE

1250 JMP osrd1

1260 :

1270 .osfile

1280 JSR datin

1290 JSR datin

1300 STX 0

1310 STY 1

1320 LDY #0

1330 LDA &8016

1340 JSR &FFDD

1350 JSR osrd1

1360 LDX #&12

1370 LDY #2

1380 JMP blkout2

1390 :

1400 .osgbpb

1410 JSR datin

1420 LDY #0

1430 LDA &8016

1440 JSR &FFD1

1450 JSR osrd1

1460 LDX #&D

1470 JMP blkout1

1480 :

1490 .cold

1500 SEI

1510 LDA #&40

1520 STA &D00

1530 LDA #&AA

1540 LDX #0

1550 LDY #&FF

1560 JSR &FFF4

1570 STX &F6

1580 STY &F7

1590 :

1600 LDX &F4

1610 LDY #7

1620 .cold1

1630 TYA

1640 JSR sidesel

1650 LDA &8000

1660 CMP &8100

1670 BNE cold5

1680 EOR #&FF

1690 STA &8000

1700 CMP &8100

1710 PHP

1720 EOR #&FF

1730 STA &8000

1740 PLP

1750 BEQ cold7

1760 :

1770 .cold5

1780 DEY

1790 BPL cold1

1800 TXA

1810 JSR sidesel

1820 BRK

1830 BRK

1840 EQUS "No E2P"

1850 BRK

1860 :

1870 .coldvec

1880 EQUW tclear

1890 :

1900 .cold7

1910 STY thisROM

1920 LDA (&F6),Y

1930 BEQ cold10

1940 CMP &8006

1950 BEQ cold16

1960 :

1970 .cold10

1980 LDX #0

1990 .cold11

2000 LDA &8000,X

2010 INX

2020 BNE cold11

2030 :

2040 .cold12

2050 LDA B%,X

2060 STA &8000,X

2070 INX

2080 BNE cold12

2090 LDA &FCE5

2100 :

2110 .cold15

2120 LDA &8000,X

2130 INX

2140 BNE cold15

2150 LDA &800D

2160 BNE cold15

2170 :

2180 .cold16

2190 LDX #coldvec MOD 256

2200 LDY #coldvec DIV 256

2210 LDA #1

2220 JSR instr

2230 :

2240 LDA #C% MOD 256:STA 0

2250 LDA #C% DIV 256:STA 1

2260 LDY #0

2270 STY langflg

2280 :

2290 .cold17

2300 JSR pload

2310 LDA 0

2320 CMP #D% MOD 256

2330 BNE cold17

2340 LDA 1

2350 CMP #D% DIV 256

2360 BNE cold17

2370 JMP (&FFFC)

2380 :

2390 .message1

2400 EQUB &D

2410 EQUS "E2P 0.10"

2420 EQUB &D

2430 EQUB 0

2440 :

2450 .message2

2460 EQUS "SECOND PROCESSOR"

2470 EQUB &D

2480 EQUB &D

2490 EQUB 0

2500 :

2510 .prnmes

2520 LDX #0

2530 .prnmes2

2540 LDA message1,X

2550 BEQ prnmes5

2560 JSR &FFE3

2570 INX

2580 BNE prnmes2

2590 .prnmes5

2600 RTS

2610 :

2620 .Rserv

2630 CPX thisROM

2640 BNE Rserv3

2650 :

2660 CMP #9

2670 BNE Rserv2

2680 BIT isec

2690 BPL Rserv2

2700 JSR prnmes2

2710 LDA #9

2720 :

2730 .Rserv2

2740 CMP #&FE

2750 .Rserv3

2760 BNE Rserv15

2770 :

2780 TYA

2790 PHA

2800 JSR &FFE7

2810 LDA #&A6

2820 LDX #0

2830 LDY #&FF

2840 JSR &FFF4

2850 STX &1D

2860 STY &1E

2870 :

2880 LDA &20B

2890 CMP #newosb DIV 256

2900 BEQ Rserv5

2910 STA oldosb+1

2920 LDA &20A:STA oldosb+0

2930 LDA #newosb DIV 256:STA &20B

2940 LDA #newosb MOD 256:STA &20A

2950 :

2960 .Rserv5

2970 LDA #&7A

2980 JSR &FFF4

2990 LDY #0

3000 STY isec

3010 CPX #&64

3020 BNE Rserv7

3030 LDA #&78

3040 JSR &FFF4

3050 JMP Rserv10

3060 :

3070 .Rserv7

3080 JSR codld

3090 LDA #&FF

3100 STA isec

3110 LDY #&EA

3120 STA (&1D),Y

3130 JSR pcomm

3140 :

3150 LDA #brk MOD 256:STA &202

3160 LDA #brk DIV 256:STA &203

3170 LDA #event MOD 256:STA &220

3180 LDA #event DIV 256:STA &221

3190 :

3200 LDA #&14

3210 LDX #6

3220 JSR &FFF4

3230 LDA #&80

3240 STA &14

3250 :

3260 LDX #message2-message1

3270 JSR prnmes2

3280 :

3290 .Rserv10

3300 PLA

3310 TAY

3320 LDA #0

3330 .Rserv15

3340 LDX &F4

3350 RTS

3360 :

3370 .newosb

3380 CMP #&84

3390 BEQ newosb2

3400 CMP #&85

3410 BNE newosb5

3420 .newosb2

3430 LDX #A% MOD 256

3440 LDY #A% DIV 256

3450 RTS

3460 :

3470 .newosb5

3480 CMP #&C8

3490 BNE newosb7

3500 TXA

3510 AND #&FD

3520 TAX

3530 LDA #&C8

3540 .newosb7

3550 JMP (oldosb)

3560 :

3570 .pcomm

3580 STA &800D

3590 .pstat

3600 NOP

3610 NOP

3620 LDA &800D

3630 BNE pstat

3640 RTS

3650 :

3660 .pselect

3670 LDA thisROM

3680 JSR sidesel

3690 JMP pstat

3700 :

3710 .sidesel

3720 STA &F4

3730 LDA #&C

3740 STA &FE05

3750 LDA &F4

3760 STA &FE05

3770 RTS

3780 :

3790 .instr

3800 CMP #&80

3810 BCC instr4

3820 CMP #&C0

3830 BCC instr3

3840 ASL &14

3850 BCS instr1

3860 CMP &15

3870 BEQ instr2

3880 CLC

3890 RTS

3900 .instr1

3910 STA &15

3920 .instr2

3930 RTS

3940 .instr3

3950 ORA #&40

3960 CMP &15

3970 BNE instr2

3980 LDA #&80

3990 STA &14

4000 LDA #&20

4010 :

4020 .instr4

4030 PHP

4040 SEI

4050 STY &13

4060 STX &12

4070 STA &16

4080 LDA &F4

4090 PHA

4100 JSR pselect

4110 :

4120 LDY #1

4130 .instr5

4140 LDA (&12),Y

4150 STA &8010,Y

4160 DEY

4170 BPL instr5

4180 :

4190 LDY &13

4200 LDA &16

4210 ORA #8

4220 .instr7

4230 JSR pcomm

4240 PLA

4250 JSR sidesel

4260 LDA &16

4270 CMP #4

4280 BNE instr9

4290 :

4300 .instr8

4310 LDA #&80

4320 STA &14

4330 JMP main

4340 .instr9

4350 PLP

4360 RTS

4370 :

4380 .esc

4390 PHP

4400 SEI

4410 LDA &F4

4420 PHA

4430 JSR pselect

4440 LDA &FF

4450 STA &8010

4460 LDA #&C0

4470 STA &16

4480 BNE instr7

4490 :

4500 .event

4510 STA &19

4520 PHP

4530 LDA &F4

4540 PHA

4550 JSR pselect

4560 STX &800E

4570 STY &800F

4580 LDA &19

4590 STA &8010

4600 LDA #&40

4610 JSR pcomm

4620 PLA

4630 JSR sidesel

4640 LDA &19

4650 PLP

4660 RTS

4670 :

4680 .warm

4690 CLI

4700 LDY #0

4710 STY 0

4720 STY &17

4730 LDA #&80

4740 STA 1

4750 STA &18

4760 LDA #&20

4770 AND &8006

4780 BEQ warm2

4790 :

4800 LDX &8007

4810 .warm1

4820 INX

4830 LDA &8000,X

4840 BNE warm1

4850 LDA &8001,X

4860 STA &17

4870 LDA &8002,X

4880 STA &18

4890 :

4900 .warm2

4910 BCS warm4

4920 BIT langflg

4930 BPL warm4

4940 LDY #&FD

4950 LDA (&1D),Y

4960 BEQ warm99

4970 .warm4

4980 LDA &F4

4990 CMP #&C

5000 BNE warm5

5010 JMP &8000

5020 :

5030 .warm5

5040 LDA #1

5050 JSR warm100

5060 .warm7

5070 JSR pload

5080 BIT 1

5090 BVS warm12

5100 :

5110 .warm10

5120 LDA 0

5130 CLC

5140 ADC &17

5150 LDA 1

5160 ADC &18

5170 CMP #&F8

5180 BCC warm7

5190 :

5200 .warm12

5210 LDA #&FF

5220 STA langflg

5230 .warm99

5240 LDA #4

5250 :

5260 .warm100

5270 PHA

5280 .warm110

5290 LDA #&FF

5300 JSR instr

5310 BCC warm110

5320 PLA

5330 LDX #&17

5340 LDY #0

5350 JMP instr

5360 :

5370 .regsout

5380 STY &8018

5390 .regso1

5400 STX &8017

5410 .regso2

5420 STA &8016

5430 .regso3

5440 ROL A

5450 STA &8015

5460 ROR A

5470 RTS

5480 :

5490 .regsin

5500 LDX &8017

5510 .regsi1

5520 LDY &8018

5530 LDA &8016

5540 RTS

5550 :

5560 .datin

5570 LDY #0

5580 .datin1

5590 LDX #0

5600 .datin2

5610 BIT &8019

5620 BVC datin5

5630 BMI datin9

5640 LDA &801A

5650 STA &700,Y

5660 BVS datin7

5670 :

5680 .datin5

5690 BPL datin2

5700 CPY #&12

5710 BCC datin6

5720 CPY #&20

5730 BCC datin7

5740 CPY #&70

5750 BCS datin7

5760 :

5770 .datin6

5780 LDA &801A

5790 STA 0,Y

5800 .datin7

5810 CLV

5820 .datin9

5830 STX &8019

5840 INY

5850 BVC datin2

5860 LDY #7

5870 RTS

5880 :

5890 .blkout

5900 LDX #4

5910 .blkout1

5920 LDY #0

5930 .blkout2

5940 STX &1A

5950 .blkout3

5960 LDX #&80

5970 .blkout5

5980 CPY &1A

5990 BEQ txtout5

6000 LDA 0,Y

6010 JSR datout

6020 INY

6030 BNE blkout5

6040 :

6050 .txtout

6060 LDX #&40

6070 LDY #0

6080 .txtout2

6090 LDA &700,Y

6100 JSR datout

6110 INY

6120 CMP #&D

6130 BNE txtout2

6140 .txtout5

6150 LDX #&C0

6160 :

6170 .datout

6180 NOP

6190 NOP

6200 BIT &801B

6210 BVS datout

6220 BMI datout

6230 STA &801C

6240 STX &801B

6250 RTS

6260 :

6270 .brk

6280 LDY #&FF

6290 .brk2

6300 INY

6310 LDA (&FD),Y

6320 STA &20,Y

6330 TAX

6340 BNE brk2

6350 TYA

6360 BEQ brk2

6370 INY

6380 STY &1A

6390 LDY #&20

6400 JSR pselect

6410 LDA #&80

6420 JSR pcomm

6430 JSR blkout3

6440 :

6450 .main

6460 LDX #&FF

6470 TXS

6480 .main2

6490 SEI

6500 LDA thisROM

6510 JSR sidesel

6520 CLI

6530 .main3

6540 LDX &8014

6550 BEQ main4

6560 STX &801D

6570 LDA #0

6580 STA &8014

6590 :

6600 .main4

6610 LDX &801D

6620 CPX &801E

6630 BEQ main9

6640 :

6650 INX

6660 CPX #vduend MOD 256

6670 BCC main7

6680 LDX #vdustart MOD 256

6690 .main7

6700 LDA &8000,X

6710 STX &801D

6720 JSR &FFEE

6730 JMP main3

6740 :

6750 .main9

6760 LDA &8013

6770 BEQ main2

6780 :

6790 LDX &801D

6800 CPX &801E

6810 BNE main3

6820 :

6830 STA osjmp-cod400+&401

6840 JSR osjmp-cod400+&400

6850 JMP main

6860 ]

6870 B%=O%-V%+U%

6880 P%=&FF00

6890 vdustart=&FF1F

6900 [OPT I        \ 2P Client code

6910 JMP warm

6920 JMP Rserv

6930 EQUB &82

6940 EQUB 8

6950 EQUB 0

6960 EQUS "(C)"

6970 EQUB 0

6980 EQUB &FF

6990 :

7000 .reset

7010 LDX #&FF

7020 TXS

7030 .reset2

7040 LDA refresh-&FF,X

7050 .reset4

7060 STA &FB00,X   \ Copy refresh code

7070 STA &FA80,X

7080 DEX

7090 CMP #&C9

7100 BNE reset2

7110 DEX

7120 BMI reset4

7130 LDA #hostcmd7 MOD 256

7140 STA &FFFC     \ Redirect Reset

7150 BNE hostcmd7

7160 :

7170 CMP #0

7180 LDA cycle+1

7190 EOR #&80

7200 STA cycle+1

7210 LDA &FF0D     \ Get host command

7220 CMP cmd       \ Check if changed

7230 STA cmd

7240 .refresh

7250 RTS

7260 .vduend       \ End of VDU queue

7270 :

7280 .irq          \ Called by &FEE5 a
ccess

7290 PHA

7300 LDA &FEE5     \ Get byte

7310 .ivec

7320 STA &200      \ Store or fetch

7330 STA &FEE5     \ Send byte

7340 INC ivec+1    \ Update address

7350 BNE irqexit

7360 INC ivec+2

7370 .irqexit

7380 PLA

7390 .anrti

7400 RTI

7410 :

7420 .cycle

7430 JSR &FB00

7440 BEQ cycle5    \ No change

7450 ORA #0

7460 .cycle5

7470 RTS

7480 :

7490 .nmi          \ Called every 1ms

7500 PHA

7510 JSR cycle     \ Refresh RAM

7520 BEQ irqexit   \ No host command

7530 :

7540 .hostcmd

7550 CMP #9

7560 BCC hostcmd2  \ cmd=8, save mem

7570 BNE hostcmd10 \ cmd<>9

7580 LDA #&8D      \ cmd=9, load mem

7590 BNE hostcmd4  \ opcode=STA

7600 :

7610 .hostcmd2

7620 LDA #&AD      \ opcode=LDA

7630 .hostcmd4

7640 STA ivec

7650 LDA &FF10     \ Get transfer add

7660 STA ivec+1

7670 LDA &FF11

7680 STA ivec+2

7690 :

7700 LDA #irq MOD 256

7710 STA &FFFE     \ Set IRQ vector

7720 LDA #anrti MOD 256

7730 STA &FFFA     \ Ignore NMIs

7740 BCS hostcmd6

7750 BRK           \ Trigger P->H

7760 NOP

7770 :

7780 .hostcmd6

7790 CLI

7800 .hostcmd7

7810 LDA #0

7820 STA &FF0D

7830 .hostcmd8

7840 JSR cycle

7850 BNE hostcmd   \ Process command

7860 BEQ hostcmd8  \ Loop back

7870 .hostcmd10

7880 JMP hostcmd12

7890 :

7900 .regsin10

7910 LDY &FF18

7920 .regsin11

7930 LDX &FF17

7940 .regsin12

7950 LDA &FF15

7960 ROR A

7970 LDA &FF16

7980 RTS

7990 :

8000 .irq5

8010 STA &FC

8020 PLA

8030 ORA #4

8040 PHA

8050 AND #&10

8060 BEQ irq7      \ Not BRK, exit

8070 :

8080 TXA

8090 PHA

8100 TSX

8110 LDA &103,X    \ Pop address

8120 CLD

8130 SEC

8140 SBC #1

8150 STA &FD

8160 LDA &104,X

8170 SBC #0

8180 STA &FE

8190 PLA

8200 TAX

8210 LDA &FC

8220 JMP (&202)    \ Jump to BRKV

8230 .irq7

8240 LDA &FC

8250 RTI

8260 :

8270 .evjmp

8280 JMP (&220)

8290 :

8300 JMP (&21C)

8310 JMP (&21A)

8320 JMP (&218)

8330 JMP (&216)

8340 JMP (&214)

8350 JMP (&212)

8360 JMP (&210)

8370 CMP #&D

8380 BNE &FFEE

8390 LDA #&A

8400 JSR &FFEE

8410 LDA #&D

8420 JMP (&20E)

8430 JMP (&20C)

8440 JMP (&20A)

8450 JMP (&208)

8460 EQUW anrti

8470 EQUW reset

8480 EQUW anrti

8490 ]

8500 C%=O%-V%+U%

8510 P%=&F800

8520 [OPT I

8530 .tclear

8540 LDA &FF1E     \ Clear VDU queue

8550 STA &FF14

8560 LDA #0

8570 STA &FF13     \ Clear command

8580 STA &FF19     \ Clear status

8590 STA &FF1B     \ Clear status

8600 :

8610 .tclear10

8620 LDA #irq5 MOD 256

8630 STA &FFFE

8640 LDA #nmi MOD 256

8650 STA &FFFA

8660 :

8670 .tclear20

8680 LDA #0

8690 STA &FF0D

8700 RTS

8710 :

8720 .hostcmd12

8730 CMP #&C

8740 BNE hostcmd15 \ Not execute

8750 LDX #&FF

8760 TXS           \ Reset stack

8770 LDA &FF11     \ Get address

8780 PHA

8790 STA osbtab+2  \ Set memtop

8800 STA osbtab+3

8810 LDA &FF10

8820 PHA

8830 SEI

8840 PHP

8850 JSR tclear

8860 LDA #1

8870 RTI            \ Jump to code

8880 :

8890 .hostcmd15

8900 CMP #&28

8910 BEQ hostcmd92  \ Start/stop

8920 BCC hostcmd92

8930 :

8940 CMP #&FF

8950 BNE hostcmd20  \ Not reset

8960 TAX

8970 TXS            \ Clear stack

8980 INX

8990 STX &FF

9000 LDX #&23

9010 STX &FF1E      \ Clear VDU queue

9020 .hostcmd17

9030 LDA vectab,X   \ Init vectors

9040 STA &200,X

9050 DEX

9060 BPL hostcmd17

9070 :

9080 .hostcmd18A

9090 JSR tclear

9100 .hostcmd18

9110 JMP hostcmd18  \ Wait for NMI/IRQ

9120 :

9130 .hostcmd20

9140 CMP #&C0

9150 BNE hostcmd25  \ Not escape

9160 LDA &FF10

9170 STA &FF        \ Set escflg

9180 BMI hostcmd92  \ Clear VDU queue

9190 :

9200 .hostcmd90

9210 LDA &FFFA

9220 CMP #anrti MOD 256

9230 BNE hostcmd95

9240 JMP hostcmd7

9250 :

9260 .hostcmd92

9270 JSR tclear10

9280 PLA

9290 RTI

9300 :

9310 .hostcmd95

9320 JSR tclear20

9330 PLA

9340 RTI

9350 :

9360 .hostcmd25

9370 CMP #&40

9380 BNE hostcmd30  \ Not event

9390 TXA

9400 PHA

9410 TYA

9420 PHA

9430 LDX &FF0E      \ Get regs

9440 LDY &FF0F

9450 LDA &FF10

9460 JSR evjmp

9470 PLA

9480 TAY

9490 PLA

9500 TAX

9510 JMP hostcmd90

9520 :

9530 .hostcmd30     \ Host BRK

9540 TXA

9550 PLA

9560 PHA

9570 PHA

9580 JSR tclear

9590 LDX #0

9600 STX &FD

9610 LDY #&FD

9620 STY &FE

9630 JSR datin10

9640 PLA

9650 TAY

9660 PLA

9670 TAX

9680 PLA

9690 STA &FC

9700 JMP (&202)

9710 :

9720 .rdch

9730 LDA #osrdch MOD 256

9740 JSR oscomm

9750 JMP regsin12

9760 :

9770 .cli

9780 LDA #oscli MOD 256

9790 STA &FF13

9800 JSR txtout10

9810 JMP osstat

9820 :

9830 .byte

9840 CMP #&82

9850 BCC byte15

9860 CMP #&86

9870 BCS byte15

9880 TAX

9890 LDY osbtab-&82,X

9900 LDX #0

9910 .anrts

9920 RTS

9930 :

9940 .osbtab

9950 EQUB 0

9960 EQUB 8

9970 EQUB &80

9980 EQUB &80

9990 :

10000 .byte15

10010 CMP #&DA

10020 BNE byte20

10030 TXA

10040 BNE byte18

10050 TYA

10060 BNE byte18

10070 LDA &FF1E

10080 STA &FF14

10090 .byte18

10100 LDA #&DA

10110 .byte20

10120 JSR regso10

10130 LDA #osbyte MOD 256

10140 JSR oscomm

10150 JMP regsin10

10160 :

10170 .word

10180 STA &FF16

10190 LDA #osword MOD 256

10200 STA &FF13

10210 LDA &FF16

10220 BNE oswd15

10230 LDA #5

10240 JSR blkout10

10250 JSR osstat

10260 JSR regsin12

10270 BCS anrts

10280 LDY #0

10290 LDA (blk),Y

10300 TAX

10310 INY

10320 LDA (blk),Y

10330 TAY

10340 JMP datin10

10350 :

10360 .oswd15

10370 STX rgX

10380 CMP #&E

10390 BCC oswd17

10400 LDA #&E

10410 :

10420 .oswd17

10430 TAX

10440 LDA oswifrm-1,X

10450 STA &FF18

10460 LDA oswito-1,X

10470 STA &FF17

10480 LDA oswoto-1,X

10490 LDX rgX

10500 JSR blkout10

10510 JSR osstat

10520 LDA &FF18

10530 AND #&1F

10540 JSR datin11

10550 JMP blkout19

10560 :

10570 .args

10580 JSR regso10

10590 LDA #osargs MOD 256

10600 STA &FF13

10610 LDY #0

10620 LDA #4

10630 CLD

10640 JSR blkout10

10650 JSR osstat

10660 JSR datin10

10670 JMP regsin10

10680 :

10690 .bget

10700 STY &FF18

10710 LDA #osbget MOD 256

10720 .bget12

10730 JSR oscomm

10740 JMP regsin12

10750 :

10760 .bput

10770 JSR regso11

10780 LDA #osbput MOD 256

10790 BNE bget12

10800 :

10810 .find

10820 JSR regso10

10830 PHA

10840 LDA #osfind MOD 256

10850 STA &FF13

10860 PLA

10870 BEQ find20

10880 JSR txtout10

10890 :

10900 .find20

10910 JSR osstat

10920 JMP regsin10

10930 :

10940 .file

10950 STA &FF16

10960 LDA #osfile MOD 256

10970 STA &FF13

10980 LDA #&12

10990 JSR blkout10

11000 LDY #0

11010 LDA (blk),Y

11020 TAX

11030 INY

11040 LDA (blk),Y

11050 TAY

11060 JSR txtout10

11070 JSR osstat

11080 LDX blk

11090 LDY blk+1

11100 LDA #2

11110 .file15

11120 JSR datin11

11130 LDX blk

11140 LDY blk+1

11150 JMP regsin12

11160 :

11170 .gbpb

11180 STA &FF16

11190 LDA #osgbpb MOD 256

11200 STA &FF13

11210 LDA #&D

11220 JSR blkout10

11230 JSR osstat

11240 CLD

11250 LDA #0

11260 BEQ file15

11270 :

11280 .oscomm

11290 STA &FF13

11300 .osstat

11310 LDA &FF13

11320 BNE osstat

11330 RTS

11340 :

11350 .regso10

11360 STX &FF17

11370 .regso11

11380 STY &FF18

11390 STA &FF16

11400 RTS

11410 :

11420 .txtout10

11430 STX txt

11440 STY txt+1

11450 LDX #&40

11460 LDY #0

11470 .txtout12

11480 LDA (txt),Y

11490 JSR datout10

11500 INY

11510 CMP #&D

11520 BNE txtout12

11530 .txtout15

11540 LDX #&C0

11550 :

11560 .datout10

11570 BIT &FF19

11580 BVS datout10

11590 BMI datout10

11600 STA &FF1A

11610 STX &FF19

11620 .datout15

11630 RTS

11640 :

11650 .blkout10

11660 STA num

11670 .blkout11

11680 STX blk

11690 STY blk+1

11700 LDX #&80

11710 LDY #0

11720 .blkout15

11730 CPY num

11740 BEQ blkout17

11750 LDA (blk),Y

11760 JSR datout10

11770 INY

11780 BNE blkout15

11790 :

11800 .blkout17

11810 JSR txtout15

11820 .blkout19

11830 LDX blk

11840 LDY blk+1

11850 RTS

11860 :

11870 .datin10

11880 LDA #0

11890 .datin11

11900 STX rpl

11910 STY rpl+1

11920 TAY

11930 LDX #0

11940 .datin12

11950 BIT &FF1B

11960 BVC datin15

11970 BMI datin19

11980 BPL datin17

11990 :

12000 .datin15

12010 BPL datin12

12020 .datin17

12030 LDA &FF1C

12040 STA (blk),Y

12050 CLV

12060 :

12070 .datin19

12080 STX &FF1B

12090 INY

12100 BVC datin12

12110 RTS

12120 :

12130 .wrch

12140 STX tmp

12150 LDX &FF1E

12160 INX

12170 CPX #vduend MOD 256

12180 BCC wrch15

12190 LDX #vdustart MOD 256

12200 .wrch15

12210 CPX &FF1D

12220 BEQ wrch15

12230 :

12240 STA &FF00,X

12250 STX &FF1E

12260 LDX tmp

12270 RTS

12280 :

12290 .oswoto

12300 EQUB 0

12310 EQUB 5

12320 EQUB 0

12330 EQUB 5

12340 EQUB 4

12350 EQUB 5

12360 EQUB 8

12370 EQUB &E

12380 EQUB 4

12390 EQUB 1

12400 EQUB 1

12410 EQUB 5

12420 EQUB 0

12430 EQUB &10

12440 :

12450 .oswifrm

12460 EQUB &20

12470 EQUB &20

12480 EQUB &20

12490 EQUB &20

12500 EQUB &24

12510 EQUB &20

12520 EQUB &20

12530 EQUB &20

12540 EQUB &24

12550 EQUB &21

12560 EQUB &21

12570 EQUB &20

12580 EQUB &20

12590 EQUB &20

12600 :

12610 .oswito

12620 EQUB &25

12630 EQUB &20

12640 EQUB &25

12650 EQUB &20

12660 EQUB &25

12670 EQUB &20

12680 EQUB &20

12690 EQUB &20

12700 EQUB &25

12710 EQUB &29

12720 EQUB &25

12730 EQUB &20

12740 EQUB &28

12750 EQUB &30

12760 :

12770 .vectab

12780 EQUW anrts

12790 EQUW hostcmd18A

12800 EQUW irq7

12810 EQUW irq7

12820 EQUW cli

12830 EQUW byte

12840 EQUW word

12850 EQUW wrch

12860 EQUW rdch

12870 EQUW file

12880 EQUW args

12890 EQUW bget

12900 EQUW bput

12910 EQUW gbpb

12920 EQUW find

12930 EQUW anrts

12940 EQUW anrts

12950 EQUW anrts

12960 :

12970 ]

12980 D%=O%-V%+U%

12990 P%=D%

13000 [OPT I

13010 .langflg

13020 EQUB 0

13030 .isec

13040 EQUB 0

13050 .oldosb

13060 EQUW 0

13070 .thisROM

13080 EQUB 0

13090 ]

13100 NEXT

13110 PRINT"IF YOU ARE USING TAPES,"

13120 PRINT"INSERT CASSETTE THAT WILL H
OLD RUN CODE."

13130 OSCLI"SAVE E2PCODE FFFF"+STR$~V%+
" +"+STR$~(D%-A%)+" FFFF"+STR$~U%+" FFF
F"+STR$~U%

13140 VDU26:CLS